NEPAL COLLEGE OF INFORMATION TECHNOLOGY

Pokhara University

LAB REPORT : Basic Discrete-Time Signals

| Course | Data Communication (CMP-335) |
| --- | --- |
| Experiment No. | 2 |
| Experiment Title | Basic Discrete-Time Signals |
| Name | Suraj Kumar Jha |
| Roll No. | 231537 |
| Program | B.E. Information Technology, 6th Semester |

# 1. Objective

- To understand and generate basic discrete-time signals using MATLAB.

- To plot and visually analyze the unit step, unit impulse, ramp, signum, and exponential (increasing and decreasing) signals.

- To observe the effect of signal parameters on the shape and amplitude of discrete-time signals.

# 2. Theory

A discrete-time signal is a sequence of numbers defined only at discrete instants of time, usually denoted as x(n), where n is an integer representing the sample index. Unlike continuous-time signals, discrete-time signals are undefined between sample points. These signals form the foundation of digital signal processing and are used to model, analyze, and test the behaviour of discrete-time systems. The basic or elementary discrete-time signals discussed in this experiment are building blocks that can be combined to represent more complex signals.

## 2.1 Unit Step Signal

The unit step signal, denoted u(n), is defined as:

u(n) = 1, for n ≥ 0 u(n) = 0, for n < 0

It has a constant amplitude of 1 for all non-negative values of n and is zero for negative n. The unit step signal is widely used to test the response of a system to a sudden, sustained input, and is analogous to switching on a constant signal at n = 0.

## 2.2 Unit Impulse Signal

The unit impulse (or unit sample) signal, denoted δ(n), is defined as:

δ(n) = 1, for n = 0 δ(n) = 0, for n ≠ 0

The unit impulse is the most fundamental discrete-time signal. Every discrete-time signal can be expressed as a weighted sum of time-shifted impulses, which makes the impulse response of a system central to the analysis of linear time-invariant (LTI) systems through convolution.

## 2.3 Ramp Signal

The unit ramp signal, denoted r(n), is defined as:

r(n) = n, for n ≥ 0 r(n) = 0, for n < 0

The ramp signal increases linearly with n for non-negative indices and is zero elsewhere. It can also be obtained by multiplying the index n with the unit step signal, i.e., r(n) = n·u(n), and is commonly used to study the response of systems to linearly increasing inputs.

## 2.4 Signum Signal

The signum function, denoted sgn(n), indicates the sign of n and is defined as:

sgn(n) = 1, for n > 0 sgn(n) = 0, for n = 0 sgn(n) = -1, for n < 0

The signum signal is closely related to the unit step signal through the relation sgn(n) = 2u(n) - 1 (with a suitable definition at n = 0), and it is useful for representing polarity or direction in a signal.

## 2.5 Exponential Signals

A discrete-time exponential signal has the general form x(n) = A·e^(αn), where A is the amplitude and α determines the rate of growth or decay. When α > 0, the signal is an exponentially increasing signal, and its amplitude grows as n increases. When α < 0, the signal is an exponentially decreasing (decaying) signal, and its amplitude reduces towards zero as n increases. Exponential signals are important because they appear naturally in the analysis of physical systems such as charging/discharging circuits, population growth or decay models, and the natural response of linear systems.

# 3. Apparatus / Software Used

- MATLAB (R2023a or equivalent) with the Signal Processing toolbox functions stem, subplot, and exportgraphics.

- A computer system capable of running MATLAB.

# 4. Procedure

- Define the discrete time index vector n ranging from -5 to 5.

- Generate the unit step, unit impulse, and ramp signals using logical and arithmetic operations on n.

- Generate the signum signal using MATLAB's built-in sign() function.

- Generate the exponentially increasing and decreasing signals using the exp() function with positive and negative rate constants respectively.

- Plot each signal using the stem() function, which is appropriate for discrete-time (sampled) data, arranging related signals together using subplot().

- Label each plot with an appropriate title, x-axis (Time Index n), and y-axis (Amplitude), and enable the grid for readability.

- Export the resulting figures as high-resolution PNG images using exportgraphics().

# 5. MATLAB Code

clc;

clear;

close all;

%% ==========================================================

% Signal Generation

%% ==========================================================

n = -5:5;

% Unit Step Signal

unit_step = (n >= 0);

% Unit Impulse Signal

impulse = (n == 0);

% Ramp Signal

ramp = n .* unit_step;

% Signum Signal

sgn_signal = sign(n);

% Exponential Increasing Signal

exp_inc = 4*exp(0.2*n);

% Exponential Decreasing Signal

exp_dec = 4*exp(-0.2*n);

%% ==========================================================

% Figure 1 : Unit Step, Impulse and Ramp

%% ==========================================================

figure;

subplot(3,1,1)

stem(n,unit_step,'filled','LineWidth',1.5)

grid on

xlabel('Time Index (n)')

ylabel('Amplitude')

title('Unit Step Signal')

ylim([-0.5 1.5])

subplot(3,1,2)

stem(n,impulse,'filled','LineWidth',1.5)

grid on

xlabel('Time Index (n)')

ylabel('Amplitude')

title('Unit Impulse Signal')

ylim([-0.5 1.5])

subplot(3,1,3)

stem(n,ramp,'filled','LineWidth',1.5)

grid on

xlabel('Time Index (n)')

ylabel('Amplitude')

title('Ramp Signal')

exportgraphics(gcf,'Figure1_BasicSignals.png','Resolution',300);

%% ==========================================================

% Figure 2 : Signum and Exponential Signals

%% ==========================================================

figure;

subplot(3,1,1)

stem(n,sgn_signal,'filled','LineWidth',1.5)

grid on

xlabel('Time Index (n)')

ylabel('Amplitude')

title('Signum Signal')

ylim([-2 2])

subplot(3,1,2)

stem(n,exp_inc,'filled','LineWidth',1.5)

grid on

xlabel('Time Index (n)')

ylabel('Amplitude')

title('Exponentially Increasing Signal')

subplot(3,1,3)

stem(n,exp_dec,'filled','LineWidth',1.5)

grid on

xlabel('Time Index (n)')

ylabel('Amplitude')

title('Exponentially Decreasing Signal')

exportgraphics(gcf,'Figure2_ExponentialSignals.png','Resolution',300);

%% ==========================================================

% End of Program

%% ==========================================================

disp('--------------------------------------------');

disp('Basic Discrete-Time Signals Plotted');

disp('Figures exported successfully.');

disp('--------------------------------------------');

# 6. Results

The MATLAB program was executed successfully for the time index n = -5 to 5, and the following discrete-time signals were plotted.

Figure 1: Unit Step, Unit Impulse, and Ramp Signals

Figure 2: Signum, Exponentially Increasing, and Exponentially Decreasing Signals

# 7. Discussion

From Figure 1, the unit step signal is observed to be zero for all n < 0 and constant at 1 for n ≥ 0, confirming its role as an on/off switching signal. The unit impulse signal shows a single non-zero sample of amplitude 1 at n = 0, consistent with its definition as an idealized, instantaneous signal. The ramp signal increases linearly for n ≥ 0, exactly matching n·u(n), and remains zero for negative n.

From Figure 2, the signum signal takes the value -1 for n < 0 and +1 for n > 0, clearly showing the sign-detecting behaviour of the function. The exponentially increasing signal (α = 0.2) grows steadily with n, while the exponentially decreasing signal (α = -0.2) decays towards zero as n increases, both starting from the same amplitude of 4 at n = 0. These results confirm the theoretical definitions of each elementary discrete-time signal and demonstrate how the sign of the exponent controls growth versus decay.

# 8. Conclusion

In this experiment, the basic discrete-time signals — unit step, unit impulse, ramp, signum, and exponential (increasing and decreasing) — were successfully generated and plotted using MATLAB. The plotted waveforms matched the theoretical expectations for each signal type, reinforcing the fundamental concepts of discrete-time signal representation that form the basis for the analysis of discrete-time systems.